The Journal of China Universities of Posts and Telecommunications

Vol. 4, No. 1, Jun. 1997

Chaotic Spread-Spectrum Communication Using Discrete-Time Synchronization*

Wang  Hai  and  Hu  Jiandong

(Department of Telecommunications Engineering, Beijing University of Posts and Telecommunications, Beijing 100088, P. R. China)

Abstract  The statistic characteristics of chaotic sequences generated by improved logistic-map are analyzed and it is found that improved logistic-map chaotic sequences have good correlation and they can be  used as address sequences in spread-spectrum communication. The discrete-time synchronization of Hénon map is immediate. We apply the discrete-time synchronization to chaotic spread-spectrum communication and propose an original communication scheme. The simulation shows that the application is successful.

Key Words  chaotic spread-spectrum communication, discrete-time synchronization

1  Introduction

Chaos is a deterministic, random-like process found  in non-linear, dynamical system, which is non-period, non-converging and bounded.  Moreover, it has a very sensitive dependence upon its initial condition and parameter [1]. Chaotic signals can be used in communication.

A chaotic map is a discrete-time dynamical system

xk+1=f(xk),0<xk<1,k=0,1,2,L

running in chaotic state. The chaotic sequence

xk:k=0,1,2,L

can be used as spread-spectrum sequence in place of PN sequence in conventional CDMA DS/SS communication systems [2]. Chaotic sequences are uncorrelated when their initial values are different, so in chaotic spread-spectrum systems, a user corresponds to an initial value.

In addition, it is possible to synchronize two copies of a discrete-time chaotic system, in the sense that their state trajectories tend asymptotically to be identical when one system is suitably driven by the other. If the driving signal is selected appropriately, the discrete-time synchronization will be immediate [3].

In CDMA spread-spectrum system, how to attain the transmitter-receiver synchronization im-mediately is a problem. We try to apply the discrete-time synchronization to chaotic spread-spectrum communication systems in order to solve this problem, and we propose an original communication scheme.

2  Improved Logistic-Map Chaotic

Sequence

Let’s consider improved logistic-map discrete-time systems. Improved logistic-map is defined by:

xk+1=f(xk)=1−2(xk)2,xkÎ(−1,1)         (1)

The chaotic sequence , xk:k=0,1,2,L =fk(x0):k=0,1,2,L, generated by im-proved logistic-map, is neither periodic nor conver-ging, and sensitively dependent on initial value.

The chaotic sequences are random-like, so probability and statistics can be used in discussing their characteristics. Reference [4]  proved  density function of points in sequence generated by Eq.(1) is

ρ(x)=1p1−x2    (0<x<1)   0               (else)	(2)

ρ(x) does not depend on initial point  . Namely, the chaotic system is ergodic.

Applying density function ρ(x)，we can get：

Property 1  The average of chaotic sequence is

x=limN→¥1Ni=0N−1xi=xρ(x)dx=0 	       (3)

Property 2  The auto-correlation function of chaotic sequence is

ac(m)=limN→¥1Ni=0N−1(xi−x)(xi+m−x)xfm(x)ρ(x)dx−x2         =0.50(m=0)(m¹0)  (4)

Property 3  If  two  chaotic sequences  begin-ning  with  different initial values x10 and x20 are not overlapping, the cross-correlation function of these two sequences is

cc12(m)=limN→¥1Ni=0N−1(x1i−x)(x2(i+m)−x)       =x1fm(x2)ρ(x1,x2)dx1dx2−x2            =0	(5)

From properties 1, 2, and 3，we know that the chaotic sequence, xk:k=0,1,2,L, is identical with white noise whose average is zero. Hence, improved logistic-map chaotic sequence is suitable to be used as spread-spectrum address sequence.

3  Discrete-Time Synchronization

We consider Hénon map, a two-dimensional extension of the logistic-map:

x1k+1=1−αx12k+x2kx2k+1=βx1k	(6)

This map exhibits a chaotic behavior in a large neighborhood of the parameter values α=1.4  and  β=0.3 .

We assume Eq.(6) to be the transmitter and we select its output  as

yk=x1k	           (7)

then the receiver equations are

x1k+1=1−αy2k+x2kx2k+1=βyk	(8)

and the synchronization errors are

Dx1k+1=Dx2kDx2k+1=0	                (9)

So we can see that the errors will reach exactly zero in two steps regardless of their initial values, i.e., chaotic systems achieve synchronization. Due to the deterministic nature of chaotic motions, once synchronization has been achieved,  the two systems will remain synchronized.

In addition,  we find that the orbit of x1 and x2 is on interval of ( —2, 2 ).

We can use this immediate synchronization in chaotic spread-spectrum multiple-access communi-cation systems.

4  Applying Discrete-Time Synchro-nization to Chaotic Spread-Spectrum Communication System

In chaotic DS/SS communication systems,  the detection of the data sequence is also based on correlation of the received signal with the chaotic sequence of  the receiver. Before transmitting data information,  the transmitter and the receiver must be synchronized. The transmitter-receiver synchroni-zation may be attained in a number of ways, such as ranging from absolute time measurement and periodic transmission of pre-defined synchronizing sequence. Here, we propose a new scheme.

In order to simplify the case, we consider that in a DS/SS communication system, there are one base station and more than one mobile terminals. The base station has two chaotic maps: Eqs.(1) and (6), each terminal also has two chaotic maps: Eqs.(1) and (8). Eqs.(6) and (8) are used for synchronization, Eq.(1) is used to generate spread-spectrum sequence.

(1) The base station sends [ y(0), y(1) ] to all terminals as driven signal to synchronize terminals. As described previously, after two steps, the termi-nals will be synchronized with the base station, i.e., x1k=x1k,x2k=x2k,(k=2,3,4,L).

(2) After synchronization, the base station communicates with terminals using distinct chaotic spread-spectrum sequences.  In detail,

(a) Between the base station and terminal 1 , the chaotic spread-spectrum sequence is generated along with its initial value x0=x12/2.

(b) Between the base station and terminal 2, the chaotic spread-spectrum sequence is generated along with its initial value x0=x13/2.

(3) The receivers generate local chaotic se-quences and demodulate received signals by cor-relation.

So, we can see that terminals synchronize with the base station immediately. The communication between terminals and the base station is simul-taneous.  In addition,  each initial value of Eq.(6). corresponds to a base station.

5   Example

We consider a simple system consisting of one base station and two terminals,  assuming that the initial value of Eq.(6) in the base station: x10=0.1 and x20=0.1,  the initial value of Eq.(8) in terminal 1 : x10=0.5 and x20=0.5, and the initial value of Eq.(8) in terminal 2 : x10=0.9 and x20=0.9.   We also assume a data bit contains 2048 chips.  The synchronized results are depicted in Fig.1. Assuming the data stream in which base station sends to terminal 1 is ( 0,1,0,1,0 ), then the demodulated result of  terminal 1 before decision is as shown in Fig.2.  We find the result is good. The chaotic spread-spectrum sequences of terminal 1 and terminal 2 are different, so terminal 2 cannot demodulate the data transmitted from the base station to terminal 1 correctly, the result is shown in Fig.3.

Fig.1 Synchronized results

Fig.2 Demodulated result of terminal 1

Fig.3 Demodulated result of terminal 2

6   Conclusion

Here, we discuss discrete-time synchronization and chaotic spread-spectrum, and propose an original communication scheme.  Experiment results show that the synchronization of this system is immediate and the communication result is correct and reliable.

References

- H. G. Schuster.  Deterministic Chaos, An Intro-duction.  Second Revised Edition, Physick- Verlag GmnH, D-6940 Weinheim, Federal Republic of Germany, 1988

- Ghobad Heidari-Bateri, Clare D.McGillem.  A Chaotic Direct-Sequence Spread Spectrum Communi-cation System, IEEE Trans. on Communications, 1994, 42(2/3/4): 1524～1527

- A. De Angeli, R. Genesio, A. Tesi.  Dead-Beat Chaos Synchronization in Discrete-Time Systems, IEEE Trans. on Circuits and Systems I: Fundamental Theory and Applications, 1995, 42(1)

- Wang Hai, Hu Jiandong.  The Improved Logistic-Map Chaotic Spread-Spectrum Sequence.  Journal of China Institute of Communications, will be published.

Wang Hai  Born in Jiangxi Province, on Aug. 7, 1971. He received his B. E. degree in Telecommunications Engi-neering from Beijing University of Posts and Telecommunications in 1993. He is currently a candidate for doctor degree in department of Telecommunications Engineering there. His current interest is chaotic CDMA systems.

Hu Jiandong  Professor of Beijing University of Posts and Telecommunications, he received his B. S. degree in the department of Electrical Engineering of Nankai University. His main interest is in spread-spectrum communications, mobile communications, circuits, and systems.

From p.54

Xia Youshen  Received M.S. degree at Nanjing University in 1989,  and now an associate  professor of  NUPT. His interest includes computing math., applied math. and artificial neural networks. He has published more than 30 papers.

Wu Xinyu  Born in Jiangsu, China, in 1939, professor and IEEE senior member. He was a Visiting Scholar in Depart-ment of  Electrical  Engineering  and  Computer  Science,

University of Illinois at Chicago(1986~1987,1994) and University of California at Berkeley(1993). He is the author of more than 80 technical papers and four books. His current interest is in circuit theory, applied graph theory, control theory, VLSI circuit layout and neural networks.

*    Manuscripture received May 12, 1996

*  This paper is supported by China National Nature Science Foundation, University Doctor Discipline Scientific Research Special Fund, and BUPT-BNR Telecommunication R&D Center.