file: LISTINGS.DOC

LISTINGS OF MAXIM SPICE MACROMODELS

-----------------------------------

REV.06      - JAN 02, 1996

Op-amps and Comparators on this diskette are grouped into families

with similar characteristics. These families appear in files named

MAXxxx.FAM where MAXxxx is the first part number of the family.

Locate op-amp and comparator types in either the FAMILY LISTINGS

or ALPHANUMERIC LISTINGS shown below.

You can use single .FAM files in your simulations as needed or

concatenate all families into one file using:

COPY MAX402.FAM+MAX406.FAM+(etc.) MAXOPAMP.MOD

-----------------------------------

FAMILY LISTINGS

CONTAINED

DEVICE    IN FAMILY

TYPE        FILE         OP-AMP/COMPARATOR FAMILY DESCRIPTION

--------------------------------------------------------------

-      MAX402.FAM     High-speed micropower

MAX402                   "    2MHz GBW,  75uA IDD, unity-gain

MAX403                   "   10MHz GBW, 375uA IDD, unity-gain

MAX438                   "    6MHz GBW,  75uA IDD, Av=5

MAX439                   "   25MHz GBW, 375uA IDD, Av=5

-      MAX406.FAM    1.2uA IDD, 2.5V-10V supply, 10pA Ib

MAX406A                  "   0.5mV Vos, unity-gain or Av=2

MAX406B                  "   2.0mV Vos, unity-gain or Av=2

MAX407                   "   Dual, 3mV Vos, unity-gain stable

MAX409A                  "   0.5mV Vos, Av=10

MAX409B                  "   2.0mV Vos, Av=10

MAX417                   "   Dual, 3.0mV Vos, Av=10

MAX418                   "   Quad, 4.0mV Vos, unity-gain stable

MAX419                   "   Quad, 4.0mV Vos, Av=10

-      MAX410.FAM    Low-noise, 28MHz, single 5V, +/-5V supply

MAX410                   "   single, 2.4nV/rt.Hz, 250uV VOS

MAX410B                  "   single, 4.0nV/rt.Hz, 250uV VOS

MAX412                   "   dual, 2.4nV/rt.Hz, 250uV VOS

MAX412B                  "   dual, 4.0nV/rt.Hz, 250uV VOS

MAX414                   "   quad, 2.4nV/rt.Hz, 320uV VOS

MAX414B                  "   quad, 4.0nV/rt.Hz, 320uV VOS

-      MAX427.FAM     Low-noise, High-Precision Op Amps

MAX427                   "  , 2.5nV/rt.Hz, 8MHz GBP, 2.5V/uS SR

MAX437                   "  , 15uV Max VOS, 60MHz GBP, Av=5

-      MAX435.FAM     Wideband Transconductance Amplifiers

MAX435                   275MHz BW, 800V/uS SR, 300uV VOSMAX436                   200MHz BW, 850V/uS SR, 300uV VOS

-      MAX473.FAM     Guaranteed 10MHz Unity-Gain BW Op Amps

MAX473                   Single, unity-gain-stable, 15V/uS SR

MAX474                   Dual, rail-to-rail output swing

MAX475                   Quad, unity-gain-stable, +3V supply

-      MAX478.FAM     Micropower, 17uA Max, Precision Op Amps

MAX47815                 Dual, micropower op amps, 15V operation

MAX4785                  Dual, micropower op amps, 5V operation

MAX4783                  Dual, micropower op amps, 3V operation

MAX47915                 Quad, micropower op amps, 15V operation

MAX4795                  Quad, micropower op amps, 5V operation

MAX4793                  Quad, micropower op amps, 3V operation

-      MAX492.FAM     Micropower, Rail-to-Rail Op Amps

MAX492                   dual, rail-to-rail, Unity-Gain Stable

MAX494                   quad, 500KHz GBP, 150uA IDD, 100uV VOS

MAX495                   single, 2.7V to 6V supply, 150uA IDD

-      MAX921.FAM     Ultra Low-Power Comparators

MAX921                   single, 1.182V int REF, Adj HYST

-      MAX922.FAM     Ultra Low-Power Comparators

MAX922                   dual, 4uA supply current, 12uS delay

-      MAX923.FAM     Ultra Low-Power Comparators

MAX923                   dual, 1.182V int REF, Adj HYST

-      MAX924.FAM     Ultra Low-Power Comparators

MAX924                   quad, 1.182V REF, 2.5V to 11V Supply

REF                      1.182V REF, 25uA source & 15uA sink

-      MAX931.FAM     Ultra Low-Power Comparators

MAX931                   single, 1.182V int REF, Adj HYST

-      MAX932.FAM     Ultra Low-Power Comparators

MAX932                   dual, 1.182V int REF, Adj HYST

-      MAX933.FAM     Ultra Low-Power Comparators

MAX933                   dual, 1.182V int REF, Adj HYST

-      MAX934.FAM     Ultra Low-Power Comparators

MAX934                   quad, 1.182V REF, 2.5V to 11V Supply

REF                      1.182V REF, 25uA source & 15uA sink

-      MAX941.FAM     High-Speed, Low-Power Comparators

MAX941                   single, int hyst, 80nS delay

MAX942                   dual, int hyst, 80nS delay

MAX944                   quad, int hyst, 80nS delay

-      MAX907.FAM     High-Speed, Ultra Low-Power Comparators

MAX907                   dual, int hyst, 40nS propagation delay

MAX908                   quad, int hyst, 40nS propagation delay

-      MAX909.FAM

MAX909                   single, int hyst, 40nS delay, int latch

-      MAX912.FAM     Ultra-Fast, Low-Power, Precision Comps

MAX912                   dual, 10nS delay, +5V or +/-5V supply

MAX913                   single, 10nS delay, internal latch

-      MAX951.FAM     Ultra-Low-Power, Op Amp + Comp + Ref

MAX951                   Unity-Gain Stable, 7uA Isupply

-      MAX952.FAM     Ultra-Low-Power, Op Amp + Comp + Ref

MAX952                   Stable Av >=10V/V, 7uA Isupply

-      MAX953.FAM     Ultra-Low-Power, Op Amp + Comp

MAX953                   Unity-Gain Stable, 5uA Isupply

-      MAX954.FAM     Ultra-Low-Power, Op Amp + Comp

MAX954                   Stable Av >=10V/V, 5uA Isupply

-      MAX4100.FAM    500MHz, Low-Power Op Amps

MAX4100                  Single, 500MHz Unity-Gain BW

-      MAX4101.FAM    500MHz, Low-Power Op Amps

MAX4101                  Single, 200MHz -3dB BW, Av=2V/V

-      MAX4112.FAM    500MHz, Current-Mode Feedback Amps

MAX4112                  Single, 500MHz -3dB BW Av >=2V/V

-      MAX4113.FAM    500MHz, Current-Mode Feedback Amps

MAX4113                  Single, 275MHz -3dB BW Av >= 8V/V

ALPHANUMERIC LISTINGS

CONTAINED

DEVICE    IN FAMILY

TYPE        FILE         OP-AMP/COMPARATOR FAMILY DESCRIPTION

--------------------------------------------------------------

MAX402    MAX402.FAM     2MHz GBW,  75uA IDD, unity-gain

MAX403    MAX402.FAM     10MHz GBW, 375uA IDD, unity-gain

MAX406A   MAX406.FAM     1.2uA IDD, 0.5mV Vos, unity-gain or Av=2

MAX406B   MAX406.FAM     1.2uA IDD, 2.0mV Vos, unity-gain or Av=2

MAX407    MAX406.FAM     1.2uA IDD, 3mV Vos, dual, unity-gain

MAX409A   MAX406.FAM     1.2uA IDD, 0.5mV Vos, Av=10

MAX409B   MAX406.FAM     1.2uA IDD, 2.0mV Vos, Av=10

MAX410    MAX410.FAM     single, 2.4nV/rt.Hz, 250uV VOS,28MHz GBW

MAX410B   MAX410.FAM     single, 4.0nV/rt.Hz, 250uV VOS,28MHz GBW

MAX412    MAX410.FAM     dual, 2.4nV/rt.Hz, 250uV VOS, 28MHz GBW

MAX412B   MAX410.FAM     dual, 4.0nV/rt.Hz, 250uV VOS, 28MHz GBW

MAX414    MAX410.FAM     quad, 2.4nV/rt.Hz, 320uV VOS, 28MHz GBW

MAX414B   MAX410.FAM     quad, 4.0nV/rt.Hz, 320uV VOS, 28MHz GBW

MAX417    MAX406.FAM     dual, 1.2uA IDD, 3.0mV Vos, Av=10

MAX418    MAX406.FAM     quad, 1.2uA IDD, 4.0mV Vos, Av=1

MAX419    MAX406.FAM     quad, 1.2uA IDD, 4.0mV Vos, Av=10

MAX427    MAX427.FAM     single, 2.5nV/rt.HZ, 8MHz GBP, Av=1

MAX435    MAX435.FAM     275MHz BW, 800V/uS SR, 300uV VOS

MAX436    MAX435.FAM     200MHz BW, 850V/uS SR, 300uV VOS

MAX437    MAX427.FAM     single, 15uV Max VOS, 60MHz GBP, Av=5

MAX438    MAX402.FAM     6MHz GBW,  75uA IDD, Av=5

MAX439    MAX402.FAM     25MHz GBW, 375uA IDD, Av=5

MAX473    MAX473.FAM     single, unity-gain-stable, 15V/uS SR

MAX474    MAX473.FAM     dual, rail-to-rail output swing, 15V/uS

MAX475    MAX473.FAM     quad, unity-gain-stable, 15V/uS SR

MAX47815  MAX478.FAM     dual, micropower op amps, 15V operation

MAX4785   MAX478.FAM     dual, micropower op amps, 5V operation

MAX4783   MAX478.FAM     dual, micropower op amps, 3V operation

MAX47915  MAX478.FAM     quad, micropower op amps, 15V operation

MAX4795   MAX478.FAM     quad, micropower op amps, 5V operation

MAX4793   MAX478.FAM     quad, micropower op amps, 3V operation

MAX492    MAX492.FAM     dual, rail-to-rail, Unity-Gain Stable

MAX494    MAX492.FAM     quad, 500KHz GBP, 150uA IDD, 100uV VOS

MAX495    MAX492.FAM     single, 2.7V to 6V supply, 150uA IDD

MAX907    MAX907.FAM     dual, int hyst, 40nS propagation delay

MAX908    MAX907.FAM     quad, int hyst, 40nS propagation delay

MAX909    MAX909.FAM     single, int hyst, 40nS delay, int latch

MAX912    MAX912.FAM     dual, 10nS delay, +5V or +/-5V supply

MAX913    MAX912.FAM     single, 10nS delay, internal latch

MAX921    MAX921.FAM     single, 1.182V int REF, adj HYST

MAX922    MAX922.FAM     dual, 4uA supply current, 12uS delay

MAX923    MAX923.FAM     dual, 1.182V int REF, adj HYST

MAX924    MAX924.FAM     quad, 1.182V int REF, adj HYST

REF       MAX924.FAM     1.182V REF, 25uA source, 15uA sink

MAX931    MAX931.FAM     single, 1.182V int REF, adj HYST

MAX932    MAX932.FAM     dual, 1.182V int REF, adj HYST

MAX933    MAX933.FAM     dual, 1.182V int REF, adj HYST

MAX934    MAX934.FAM     quad, 1.182V int REF, adj HYST

REF       MAX934.FAM     1.182V REF, 25uA source, 15uA sink

MAX941    MAX941.FAM     single, int HYST, 80nS delay

MAX942    MAX941.FAM     dual, int HYST, 80nS delay

MAX944    MAX941.FAM     quad, int HYST, 80nS delay

MAX951    MAX951.FAM     Unity-Gain Stable, 7uA Isupply

MAX952    MAX952.FAM     Stable AV >=10V/V, 7uA Isupply

MAX953    MAX953.FAM     Unity-Gain Stable, 5uA Isupply

MAX954    MAX954.FAM     Stable AV >=10V/V, 5uA Isupply

MAX4100   MAX4100.FAM    Single, 500MHz Unity-Gain BW

MAX4101   MAX4101.FAM    Single, 200MHz -3dB BW, Av=2V/V

MAX4112   MAX4112.FAM    Single, 500MHz -3dB BW Av >=2V/V

MAX4113   MAX4113.FAM    Single, 275MHz -3dB BW Av >= 8V/V

###############################################################################################################