SYSCHK

System Checkout for your personal computer

Version 2.35a

Released January 5, 1993

Copyright 1993 by Advanced Personal Systems

All rights reserved.

SYSCHK is a program that provides valuable information about devices installed

in your system. The following are the items SYSCHK will search for :

- Computer model type including manufacturer if known

- Micro Processor including the 80486

- Detection of the older 80386 that had a 32-bit multiply bug

- Math Coprocessor type

- Detection of slave 8259 Interrupt controller

- Presence of a real time clock

- Presence of a Micro Channel bus

- System BIOS including source, date and size

- A listing of all known BIOS extensions in the system, and the

size of those BIOS

- Type of keyboard attached (84 key or 101 key) and keyboard status

- Type of mouse installed, number of buttons, and interrupt used

- Number of parallel ports in system and the base addresses

- Number of serial ports in system and the base addresses and

parameters of the port

- Type of Serial port UART chip including 16550

- Various information on a Network connection

- Hard disk size including tracks, heads, sectors and CMOS type

- Hard disk controller manufacturer

- Hard disk partition information

- Floppy disk size and parameters

- Video type, chipset, video memory, and BIOS source

- Amount of environment space used and available

- Amount of conventional memory total and available

- Amount of extended memory total and available

- Amount of expanded memory total and available, version and

location of page frame

- Amount of XMS memory

- Listing of all resident programs and the size of those programs

- Listing of all non standard device drivers installed

- Throughput speed of system, taking into account CPU speed and

wait states

- Self checking for virus detection

- Operates under Windows in all modes (SYSCHK.ICO is included

for your use with Windows).

A hardcopy printout or file of your system's information is also available

as an option.  Standard Output redirection is also available for use in

batch files by simply adding "/F" as a SysChk parameter.

For example "SYSCHK /f > MyInfo.txt" will create a file called MYINFO.TXT

or "SYSCHK /f" will display a text file to the console screen.

SCSI host adapter determination can sometimes cause the SCSI bus to hang.

If you wish to disable the host adapter determination then enter /S as a

SYSCHK parameter (SYSCHK /s).

Support questions may be submitted through CompuServe ID [70323,136].

REVISION HISTORY

----------------

0.99 - Prerelease version to public domain, all items on a single page

2.00 to 2.23 - Various Beta versions released only to Beta users

2.24 - Official Shareware version first release

2.30 - Added environment memory information.

Combined Floppy and Hard drive information on one page.

Added module on Networks.

Added chipset information on VGA video cards.

Added presence detection of DOS 5 in HMA area.

Added presence of execution under Microsoft Windows.

Added detection of serial ports not originally found by BIOS.

Added option to save information to a file in addition to printer.

Added disk controller manufacturer information.

Added a few more model byte descriptions.

Fixed DOS version lower than 3.0 announcement bug.

Fixed bug with DISK module running with DR DOS 5.0.

2.31 - Added detection of OAK Technologies Video chipset

2.32 - Added detection of CPU clock speed

Added Network information to printer output

Added dectection of 8514/A

Added standard output redirection option to the parameter line.

2.33 - Fixed missing info of mouse and network in redirected output.

Added some more information on HMA and XMS.

2.34 - Fixed incorrect info on HMA area.

Sped up initialization time.

Changed speed bar to accomodate faster systems.

Added detection of Logitech and Mouse Systems mice.

Added detection of ISA or EISA bus type.

Added detection of MODEM on serial port.

Added detection of Sound Board.

Added detection of (some) SCSI Host Adapters. (Test disabled by

entering Syschk /S)

2.35   Removed detection of Trident video boards (caused hangup).

Added Video Speed option.

Added detection of some S3 video chipsets.

Added detection of 80386SX CPU.

Added information on registration through CompuServe.

2.35a  Fixed problem with Video Speed reporting incorrect speed.

REGISTRATION

------------

The registration fee for this shareware product is $20.00. For the

$20.00 registration fee, you will be sent the new version of SYSCHK

version 3.0 for Windows when it is available.

In addition, you will also receive the Advanced Personal System's Deluxe

Utility Disk with a number of handy utilities including a menu system,

an easy to use file viewer, a CMOS viewer and editor, and lots more.

You can now register through CompuServe by entering "GO SWREG" and billing

your CompuServe account.  The registration ID# is 497.  Or if you like,

send in your name with a check or money order and your choice of diskette

size to:

Advanced Personal Systems

105 Serra Way

Suite 418

Milpitas, CA. 95035

(408) 298-3703    FAX (408) 945-0242

Note: California residents please add 8.25% sales tax.

Advanced Personal Systems (APS) hereby disclaims all warranties

relating to this software, whether express or implied, including

without limitation any implied warranties of merchantability or

fitness for a particular purpose. APS will not be liable for any

special, incidental, consequential, indirect or similar damages

due to loss of data or any other reason, even if APS or an agent

of APS has been advised of the possibility of such damages.  In

no event shall APS's liability for any damages ever exceed the

price paid for the license to use software, regardless of the form

of the claim.  The person using the software bears all risk as to

the quality and performance of the software.